Università Degli Studi Di Napoli “Federico II”

Scuola Politecnica e delle Scienze di Base

Dipartimento di Ingegneria Industriale

Tesi di Laurea Triennale in Ingegneria Aerospaziale

A Java Software for

Aircraft Flight Dynamics

Calculation

| Relatore | Candidato |
| --- | --- |
| Ch.mo PROF. Ing. Agostino De Marco | Giuseppe Scarlatella |
|  | N35/678 |

Anno accademico 2013/2014

- INDEX

SUMMARY	2

1	INTRODUCTION	3

1.1	LINEARIZED EQUATIONS SYSTEM	3

1.1.1	LONGITUDINAL AND LATER-DIRECTIONAL EQUATIONS SYSTEMS	5

1.1.2	STABILITY AND CONTROL DERIVATIVES	8

1.2	AIRCRAFT DYNAMICS	10

1.2.1	LONGITUDINAL DYNAMICS	10

1.2.2	LATERAL-DIRECTIONAL DYNAMICS	11

2	SOFTWARE	12

2.1	STABILITY DERIVATIVES CALCULATOR CLASS	13

2.1.1	STABILITY AND CONTROL DERIVATIVES	13

2.1.2	LONGITUDINAL AND LATERAL-DIRECTIONAL MATRICES	13

2.1.3	FUNCTIONS AND SUBROUTINES	15

2.2	DYNAMIC STABILITY CALCULATOR CLASS	16

2.2.1	EIGENVALUES AND EIGENVECTORS	16

2.2.2	DYNAMIC CHARACTERISTICS	17

2.3	FLIGHT DYNAMICS MANAGER CLASS	18

2.3.1	GLOBAL ENVIRONMENT	18

2.3.2	INPUT MANAGEMENT CLASSES	20

2.3.3	CONSTRUCTOR METHOD	23

2.3.4	CALCULATOR METHOD	23

2.3.5	MAIN METHOD	26

3	BOEING 747 – TEST	27

3.1	CONDITIONS OF FLIGHT	27

3.2	OUTPUT	29

3.3	FINAL REMARKS	34

BIBLIOGRAPHY	36

NOTES	36

# SUMMARY

This Java program is able to determine the characteristics of dynamic stability for small perturbations in longitudinal and lateral-directional motion of an aircraft, from a set of input data supplied by the manufacturer or previously estimated.

Small perturbations allow to linearize the equations of motion system, generating a system of linear equations. This linear system can be decomposed into two subsystems: longitudinal dynamic equations and lateral-directional dynamic equations.

The program consists of three main classes: one for the stability and control derivatives calculation and the related matrices generation, another one is for eigenvalues and eigenvectors management, as well as the free response to small perturbations characteristics, and the last one is a calculator class, executed in the main. After reading from file, the calculator class estimates derivatives and free response characteristics, saves all results in member variables and prints them on the screen simultaneously.

This program was built in a “bottom-up" perspective. Thanks to its setting from Object-Oriented Programming, it can be easily implemented within a more complex code. Its methods and its attributes can be accessed at any time and can interact with other classes mostly oriented to the complete aircraft study.

# INTRODUCTION

An aircraft is a deformable solid indeed and is not allowed to ignore the effects of its articulated subsystems, but is to be considered acceptable the hypothesis of rigid body for the study of flight dynamics.

In addition, the external forces and moments acting on the vehicle are not simple configuration and motion functions, especially aerodynamic actions.

At any given moment, they can be calculated if you know the current values of speed of the relative wind and angles of trim, but only with a certain approximation.

The level of detail necessary to determine the entity is the dominant theme of any formulation of the equations of motion.

The complete motion equations of a rigid aircraft in the atmosphere, subject to the aerodynamic actions, propulsion and weight force, are nonlinear and coupled. They can only be solved numerically and do not conceptually lend themselves to illustrate the dependence of stability and controllability characteristics of the aircraft from its geometric, inertial and aerodynamic properties.

## LINEARIZED EQUATIONS SYSTEM

Mostly we can learn about aircraft behavior by analyzing the linear approximations of the complete motion equations. The solutions of the linearized equations are valid for small perturbations around to a given flight condition of reference, and they can be analyzed with tools well known by mathematical theory of dynamical systems.

The particular formulation of linear problem will depend on the reference condition choice. The nominal condition adopted lends itself particularly to the linearization procedure. It is a longitudinally symmetrical not accelerated movement along a straight line, at constant speed and rate of steady climb (in particular, zero), assuming the Earth as flat, namely a balanced flight condition, that perfectly meets the complete equations of motion.

Figure 1.1- perturbed longitudinal motion (De Marco & Coiro, 2015)

Obviously, there are situations in which this approach has limitations, which occurs for all those motions characterized by large changes of the state variables or from not small assets and nonlinear aerodynamic effects.

Despite everything, the accuracy with which we can apply the small perturbations theory proves itself nevertheless acceptable in a wide spectrum of possible situations. This is essentially due to two circumstances:

- for a wide range of flight conditions of practical importance, aerodynamic forces and moments retain effectively a linear dependence with status and control variables;

- normal flight situations are actually perturbed motions of a certain amplitude, not infinitesimal, which correspond, indeed, to the combined effect of small linear and angular speed perturbations;

In fact, relatively small perturbation of the state variables can lead to flight conditions particularly 'violent' and this should normally be avoided.

### LONGITUDINAL AND LATER-DIRECTIONAL EQUATIONS SYSTEMS

The longitudinal and lateral-directional linearized equations systems can be put in the following simplified form:

x'LON = ALON  xLON + BLON uLON

x'LD = ALD  xLD + BLD uLD

where xLON and xLD are perturbations of the state variables vectors, while  uLON and uLD are perturbations of the control parameters vectors.

To better highlight the decoupling of longitudinal and lateral-directional motions, we rewrite the equations provided in matrix form:

x'LON--x'LD  ≈  [ALON]00[ALD] ∙ xLON--xLD  +  [BLON]00[BLD] ∙ uLON--uLD

Usually x vectors are composed by dynamic and cinematic state variables such as: u, v, w, q, p, r, XE,G, YE,G, ZE,G, θ, ϕ, ψ:

x  =  xLON--xLD  =  uwqXe,gZe,gθ--vprYe,gϕψ             ;             u = uLON--uLD = δTδe--δaδr

As anticipated, we can work in simplified hypothesis, keeping a general validity for many disparate cases. In particular we assume our nominal condition as a longitudinally symmetrical (v0=0) not accelerated movement along a straight line (p0 = q0 = r0 = 0), at constant speed and rate of steady climb (in particular, null), null angular velocity (ϕ0 = 0), null initial prow angle (ψ0 = 0) and assuming the Earth as flat.

Working under these hypotheses, our state vectors and coefficient matrices seem very simplified:

xLON   =  uwqθ             ;             uLON  = δTδe

xLD   =  rβpϕ             ;             uLD  = δaδr

Matrices  A  and  B  are usually an articulated stability and control derivatives assembling but, adopting our simplified nominal condition, many coefficients get lightened. In particular, matrices  A  and  B  get reduced to (4×4) and (4×2) order matrices, assuming their well-known structure:

ALON  =  XuXw0-gZu1-ZwZw1-ZwZq+U01-Zw-gSθ1-ZwMu+kZuMw+kZwMq+k(Zu+U0)-kgSθ0010

ALD  =  N'rN'βN'p0YrU0-1YβU0YpU0gU0L'rL'βL'p00010

BLON  =  XδTXδeZδT1-ZwZδe1-ZwMδT+kZδTMδe+kZδe00

BLD  =  N'δaN'δrYδaU0YδrU0L'δaL'δr00

### STABILITY AND CONTROL DERIVATIVES

The stability and control derivatives are the matrices A and B constituents. They derive directly from theories of aerodynamics and flight mechanics, and we generally will estimate them respectively to aircraft mass (in case of forces) or moments of inertia (in case of moments of forces). They are all reported in the following lists.

N.B.

The ALD and BLD coefficients are also known as “primed derivatives”, and they result quite different from dimensional derivatives reported in the list on the left. Primed derivatives are obtained dividing dimensional derivatives for a linear combination of their moments and
product of inertia:

i.e.

Primed derivatives consider combined effect between Roll and Yaw.

Figure 1.2 lateral-directional stability and control derivatives (De Marco & Coiro, 2015)

Figure 1.3 longitudinal stability and control derivatives (De Marco & Coiro, 2015)

## AIRCRAFT DYNAMICS

The linear equations systems can be studied with classical methods of the linear time-invariant (LTI) theory. Study the system response will not be in our interest, rather we will just evaluate the dynamic open-loop characteristics, whose validity is extended to all specific cases in the presence of external forcing.

### LONGITUDINAL DYNAMICS

It’s our interest to analyze longitudinal open-loop response to estimate basic characteristics such as damping coefficient and natural frequency. To do so, we have to determine the characteristic polynomial of longitudinally symmetrical motion, which is the characteristic polynomial of dynamic matrix ALON indeed, and, in terms of the eigenvalues ​​of the matrix, can be expressed in the form:

Δ LON(s)  = (s – λ SP)(s – λ*SP)(s – λ PH)(s – λ* PH)

{ λ SP , λ*SP } = σ SP ± i ω SP          ;          { λ PH , λ*PH } = σ PH ± i ω PH

its roots are two pairs of complex and conjugated eigenvalues (λ PH , λ*PH) and
(λ SP , λ*SP), which constitute two modal components of the dynamic, respectively known as phugoid (or long-term) and short-period modes.

Introducing natural frequency and damping coefficient:

ζξ= 11+(ωξ σξ)2           ;        ωnξ= -σξζξ

Tξ= 2πωnξ1-ζξ2    ;        t12ξ= ln2ωnξζξ

where ξ = SP, PH.

Figure 1.4 typical short-period oscillation, with changes on both α and θ (De Marco & Coiro, 2015)

### LATERAL-DIRECTIONAL DYNAMICS

Similarly, we will analyze lateral-directional open-loop response. The matrix ALD characteristic equation has the following roots:

λ ROLL = σ ROLL          ;          λ SPIRAL = σ SPIRAL

{ λ DR , λ* DR } = σ DR ± i ω DR

They are two real roots and a complex and conjugated one (λ DR , λ* DR), which constitute three modal components of the dynamic, respectively known as roll, spiral and dutch-roll modes.

Introducing natural frequency and damping coefficient for dutch-roll mode:

ζDR= 11+(ωDR σDR)2           ;        ωnDR= -σξζξ

TDR= 2πωnDR1-ζDR2    ;        t12DR= ln2ωnDRζDR

# SOFTWARE

As a software for Flight Dynamics Calculation, our program consists of a series of routines able to handle sufficiently all the information related to the aircraft dynamics, which includes the stability derivatives calculation and the system response characteristics.

The program has been written in Java programming language, that is concurrent, class-based but, mostly, object-oriented: “In the object-oriented (OO) paradigm, a program consists of interacting objects. An object encapsulates data and algorithms. Data defines the state of an object. Algorithms define the behavior of an object. An object communicates with other objects by sending messages to them.” (Sharan, 2014).

This is exactly the philosophy that our program embraces.  In fact, it is organized in three main classes, each of which treats a particular aspect of the global analysis:

- StabilityDerivativesCalc.java: calculator of stability derivatives and matrices ALON, BLON, ALD, BLD;

- DynamicStabilityCalculator.java: calculator of eigenvalues, eigenvectors and response characteristics;

- FlightDynamicsManager.java: Java Main Method (JVM), I/O operator and CalculateAll method.

Our approach is to describe briefly the function of each class and their structure, showing some excerpts of code to better contextualize.

## STABILITY DERIVATIVES CALCULATOR CLASS

The class StabilityDerivativesCalc.java employs more than forty different routines to estimate stability (and control) derivatives and to build longitudinal and lateral-directional matrices.

### STABILITY AND CONTROL DERIVATIVES

The following type of procedures is based on aerodynamics theorems and considerations on flight mechanics. We will now see an example.

EXAMPLE 2.1

Xªu_CT:                                                     XuCP= -q0SmU0(2CD0+M0∂CD∂M0)

public static double calcXªu_CT (double rho0, double surf, double mass, double u0,

double q0, double cd0, double m0, double cdM0) {

return -q0*surf*(2*cd0 + m0*cdM0)/(mass*u0);

}

Figure 2.1 - "Xªu_CT" calculation procedure

This type of method estimates all our stability and control derivatives.

### LONGITUDINAL AND LATERAL-DIRECTIONAL MATRICES

This type of procedures is designed for longitudinal and lateral-directional matrices assembling.

EXAMPLE 2.2

Building ALON matrix:

ALON  =  XuXw0-gZu1-ZwZw1-ZwZq+U01-Zw-gSθ1-ZwMu+kZuMw+kZwMq+k(Zu+U0)-kgSθ0010

public static double[][] build_A_Lon_matrix (Propulsion propulsion_system,

double rho0, double surf, double mass, double cbar, double u0, double q0,

double cd0, double m0, double cdM0, double cl0, double cdAlpha0, double gamma0,

double theta0_rad, double clAlpha0, double clAlpha_dot0, double cMAlpha0,

double cMAlpha0_dot, double clQ0, double iYY, double cM_m0, double cMq) {

double [][] aLon = new double [4][4];

// Propulsion type in the Xªu calculation

switch (propulsion_system)

{

case CONSTANT_TRUST:

aLon [0][0] = calcX_u_CT (rho0, surf, mass, u0, q0, cd0, m0,

cdM0);

break;

case CONSTANT_POWER:

aLon [0][0] = calcX_u_CP (rho0, surf, mass, u0, q0, cd0, m0,

cdM0, cl0, gamma0);

break;

default:

aLon [0][0] = calcX_u_CT (rho0, surf, mass, u0, q0, cd0, m0,

cdM0);

break;

}

double k = calcM_w_dot (rho0, mass, surf, cbar, iYY, cMAlpha0_dot)/(1 –

calcZ_w_dot (rho0, surf, mass, cbar, clAlpha_dot0));

// Construction of the Matrix [A Lon]

aLon [0][1] = calcX_w(rho0, surf, mass, u0, q0, cl0, cdAlpha0);

aLon [0][2] = 0;

aLon [0][3] = -(9.8100)*Math.cos(theta0_rad);

aLon [1][0] = calcZ_u (rho0, surf, mass, u0, q0, m0, cl0)/(1 –

calcZ_w_dot (rho0, surf, mass, cbar, clAlpha_dot0));

aLon [1][1] = calcZ_w (rho0, surf, mass, u0, q0, m0, cd0, clAlpha0)/(1 –

calcZ_w_dot (rho0, surf, mass, cbar, clAlpha_dot0));

[…]

aLon [2][3] = -k*(9.8100)*Math.sin(theta0_rad);

aLon [3][0] = 0;

aLon [3][1] = 0;

aLon [3][2] = 1;

aLon [3][3] = 0;

return aLon;

}

Figure 2.2 - "[A_Lon]" building procedure

EXAMPLE 2.3

Primed derivatives Calculation:

Figure 2.3 - "Primed Derivative" calculation for [A_Ld] and [B_Ld] assembling

// Inertia coefficient calculation

double i1 = iXZ/iXX;

double i2 = iXZ/iZZ;

// Primed Derivatives calculation

double Y_beta_1 = calcY_beta (rho0, surf, mass, u0, q0, cyBeta);

double Y_p_1 = calcY_p (rho0, surf, mass, u0, q0, bbar, cyP);

double Y_r_1 = calcY_r (rho0, surf, mass, u0, q0, bbar, cyR);

double L_beta_1 = (calcL_beta (rho0, surf, bbar, iXX, u0, q0, cLBeta) +

i1*calcN_beta (rho0, surf, bbar, iZZ, u0, q0, cNBeta))/(1-i1*i2);

[…]

### FUNCTIONS AND SUBROUTINES

In this category, we have many subroutines designed to estimate some flight characteristics, such as the dynamic pressure, recalled inside our main routines.

EXAMPLE 2.4

Dynamic Pressure:

public static double calcDynamicPressure(double rho0, double u0) {

return 0.5*rho0*Math.pow(u0,2);

Figure 2.4 - "Dynamic Pressure" calculation

}

EXAMPLE 2.5

Propulsive Regime:

Figure 2.5 - "Propulsive Regime" Enum type declaration

public enum Propulsion { CONSTANT_TRUST, CONSTANT_POWER, CONSTANT_MASS_FLOW,
 RAMJET }

We have declared a new enumeration type called Propulsion, which describes our propulsive regime. This choose influences XuCP and XδT calculation by a switch case during  ALON  and  BLON  calculation (see pag.14).

## DYNAMIC STABILITY CALCULATOR CLASS

The class DynamicStabilityCalculator.java employs two routines to build the eigenvectors and an eigenvalues matrix and other five different routines to estimate dynamic characteristics, such as damping coefficient or natural frequency.

### EIGENVALUES AND EIGENVECTORS

The eigenvalues calculation is fundamental to get the dynamic characteristics. We obtain them by using a routine named buildEigenValuesMatrix, that includes many other routines imported from a package, org.apache.commons.math3 (Apache Commons, s.d.), which allows you to generate two vectors, from matrices as ALON  and  BLON, each one containing respectively the real and imaginary roots parts from the characteristic polynomial. Finally, these are reported in a matrix (in our case 4x2) which contains on each line the real and imaginary part of each eigenvector.

EXAMPLE 2.6

Figure 2.6 - Eigenvalues Matrix composition procedure

buildEigenValuesMatrix:

public static double[][] buildEigenValuesMatrix (double aMatrix[][]) {

RealMatrix aLonRM = MatrixUtils.createRealMatrix(aMatrix);

EigenDecomposition aLonDecomposition = new EigenDecomposition(aLonRM);

double[] reEigen = aLonDecomposition.getRealEigenvalues();

double[] imgEigen = aLonDecomposition.getImagEigenvalues();

double [][] lambda_Matrix = new double [4][2];

for (int i=0 ; i < 4 ; i++) {

lambda_Matrix[i][0] = reEigen [i];

lambda_Matrix[i][1] = imgEigen [i];

}

return lambda_Matrix;

}

On each row of the matrix we have real part and imaginary part (separated).

Equally important it is to get the eigenvectors obtained through a procedure called buildEigenVector, which is recalled as many times as the number of the eigenvectors to calculate, specifying each time the index for the ith eigenvector.

EXAMPLE 2.7

Figure 2.7 – An Eigenvector composition procedure

buildEigenVector:

public static RealVector buildEigenVector (double aMatrix[][], int index) {

RealMatrix aRM = new Array2DRowRealMatrix(aMatrix);

EigenDecomposition eigDec = new EigenDecomposition(aRM);

RealVector eigVec = eigDec.getEigenvector(index);

return eigVec;

}

### DYNAMIC CHARACTERISTICS

These procedures are based on many classical methods from the linear time-invariant (LTI) theory. They are designed to calculate all the dynamic characteristics concerning an open-loop response and equally valid in all the other specific cases. They use as inputs all the elements from the ith line of the eigenvalues matrix previously calculated.

EXAMPLE 2.8

Damping coefficient:                                     ζDR= 11+(ωDR σDR)2

public static double calcZeta (double sigma, double omega) {

return Math.sqrt( 1 / ( 1 + Math.pow( omega/sigma , 2 )));

}

Figure 2.8 - "Damping Coefficient" calculation procedure

This is an exact value of mode characteristics (not approximate).

## FLIGHT DYNAMICS MANAGER CLASS

The class FlightDynamicsManager.java contains all the necessary procedures to perform the dynamic stability calculation of the aircraft, reading from file and storing the read data as global variables, performing the procedures imported from the classes previously treated and printing their results.

### GLOBAL ENVIRONMENT

A global variable is visible (hence accessible) throughout the program, unless shadowed. The set of all global variables is known as the global environment or global state. In compiled languages, global variables are generally static variables, whose extent (lifetime) is the entire runtime of the program. They are generally dynamically allocated when declared, since they are not known ahead of time but, once we will read our file, they will be reallocated with a specific value. Here is shown our global variables list.

EXAMPLE 2.9

Global Variables List:

Propulsion propulsion_system = Propulsion.CONSTANT_TRUST;  // propulsion

regime type

double rho0;		// air density

double surf;		// wing area

double mass; 	 	// total mass

double cbar; 		// mean aerodynamic chord

double bbar; 		// wingspan

double u0; 		// speed of the aircraft

double q0;              // dynamic pressure

double m0;              // Mach number

double gamma0;    	// ramp angle

double theta0_rad = Math.toRadians(gamma0); 	// Euler angle [rad] (assuming gamma0 = theta0)

double iXX; 		// lateral-directional moment of inertia (IXX)

double iYY; 		// longitudinal moment of inertia  (IYY)

double iZZ; 		// lateral-directional moment of inertia (IZZ)

double iXZ; 		// lateral-directional product of inertia (IXZ)

Figure 2.9 – “Global Variables” list (part 1 of 3)

double cd0; 		// drag coefficient at null incidence (Cdº) of the aircraft

double cdAlpha0; 	// linear drag gradient (CdAlphaº) of the aircraft

double cdM0; 		// drag coefficient with respect to Mach (ClMº) of the aircraft

double cl0; 		// lift coefficient at null incidence (Clº) of the aircraft

double clAlpha0; 	// linear lift gradient (ClAlphaº) of the aircraft

double clAlpha_dot0; 	// linear lift gradient time derivative (ClAlpha_dotº) of the aircraft

double clQ0; 		// lift coefficient with respect to q (ClQº) of the aircraft

double clM0;		// lift coefficient with respect to Mach (ClMº) of the aircraft

double clDelta_T; 	// lift coefficient with respect to delta_T (ClDelta_Tº) of the aircraft

double clDelta_E; 	// lift coefficient with respect to delta_E (ClDelta_Eº) of the aircraft

double cMAlpha0; 	// pitching moment coefficient with respect to Alpha (CmAlphaº) of the aircraft

double cMAlpha_dot0; 	// pitching moment coefficient time derivative (CmAlpha_dotº) of the aircraft

double cM_m0; 		// pitching moment coefficient with respect to Mach number

double cMq; 		// pitching moment coefficient with respect to q

double cMDelta_T; 	// pitching moment coefficient with respect to delta_T (CMDelta_Tº) of the aircraft

double cMDelta_E;	// pitching moment coefficient with respect to delta_E (CMDelta_Eº) of the aircraft

double cTfix; 		// thrust coefficient at a fixed point ( U0 = u , delta_T = 1 )

double kv; 		// scale factor of the effect on the propulsion due to the speed

double cyBeta; 		// lateral force coefficient with respect to beta (CyBeta) of the aircraft

double cyP; 		// lateral force coefficient with respect to p (CyP) of the aircraft

double cyR; 		// lateral force coefficient with respect to r (CyR) of the aircraft

double cyDelta_A; 	// lateral force coefficient with respect to delta_A (CyDelta_A) of the aircraft

double cyDelta_R; 	// lateral force coefficient with respect to delta_R (CyDelta_R) of the aircraft

double cLBeta; 		// rolling moment coefficient with respect to beta (CLBeta) of the aircraft

double cLP; 		// rolling moment coefficient with respect to a p (CLP) of the aircraft

double cLR; 		// rolling moment coefficient with respect to a r (CLR) of the aircraft

double cLDelta_A; 	// rolling moment coefficient with respect to a delta_A (CLDelta_A) of the aircraft

double cLDelta_R; 	// rolling moment coefficient with respect to a delta_R (CLDelta_R) of the aircraft

double cNBeta; 		// yawing moment coefficient with respect to a beta (CNBeta) of the aircraft

double cNP; 		// yawing moment coefficient with respect to p (CNP) of the aircraft

double cNR; 		// yawing moment coefficient with respect to r (CNR) of the aircraft

double cNDelta_A; 	// yawing moment coefficient with respect to delta_A (CNDelta_A) of the aircraft

double cNDelta_R; 	// yawing moment coefficient with respect to delta_R (CNDelta_R) of the aircraft

double x_u_CT;          // dimensional derivative of force component X with respect to "u" for Constant Thrust

Figure 2.10 - “global variables” list (part 2 of 3)

double x_u_CP;          // dimensional derivative of force component X with respect to "u" for Constant Power

double x_w;		// dimensional derivative of force component X with respect to "w"

double x_w_dot;         // dimensional derivative of force component X with respect to "w_dot"

double x_q;		// dimensional derivative of force component X with respect to "q"

double z_u;		// dimensional derivative of force component Z with respect to "u"

double z_w;		// dimensional derivative of force component Z with respect to "w"

double z_w_dot;		// dimensional derivative of force component Z with respect to "w_dot"

double z_q;		// dimensional derivative of force component Z with respect to "q"

double m_u;		// dimensional derivative of pitching moment M with respect to "u"

double m_w;		// dimensional derivative of pitching moment M with respect to "w"

double m_w_dot;		// dimensional derivative of pitching moment M with respect to "w_dot"

double m_q;		// dimensional derivative of pitching moment M with respect to "q"

double x_delta_T_CT;	// dimensional control derivative of force component X with respect to "delta_T" for Constant Thrust

double x_delta_T_CP;	// dimensional control derivative of force component X with respect to "delta_T" for Constant Power

double x_delta_T_CMF;	// dimensional control derivative of force component X with respect to "delta_T" for Constant Mass Flow

double x_delta_T_RJ;	// dimensional control derivative of force component X with respect to "delta_T" for RamJet

double x_delta_E;	// dimensional control derivative of force component X with respect to "delta_E"

double z_delta_T;	// dimensional control derivative of force component Z with respect to "delta_T"

double z_delta_E;	// dimensional control derivative of force component Z with respect to "delta_E"

double m_delta_T;	// dimensional control derivative of pitching moment M with respect to "delta_T"

double m_delta_E;	// dimensional control derivative of pitching moment M with respect to "delta_E"

double y_beta;		// dimensional derivative of force component Y with respect to "beta"

double y_p   ;		// dimensional derivative of force component Y with respect to "p"

double y_r   ;		// dimensional derivative of force component Y with respect to "r"

double l_beta;		// dimensional derivative of rolling moment L with respect to "beta"

double l_p   ;		// dimensional derivative of rolling moment L with respect to "p"

double l_r   ;		// dimensional derivative of rolling moment L with respect to "r"

double n_beta;		// dimensional derivative of yawing moment N with respect to "beta"

double n_p   ;		// dimensional derivative of yawing moment N with respect to "p"

double n_r   ;		// dimensional derivative of yawing moment N with respect to "r"

double y_delta_A;	// dimensional control derivative of force component Y with respect to "delta_A"

double y_delta_R;	// dimensional control derivative of force component Y with respect to "delta_R"

double l_delta_A;	// dimensional control derivative of rolling moment L with respect to "delta_A"

double l_delta_R;	// dimensional control derivative of rolling moment L with respect to "delta_R"

double n_delta_A;	// dimensional control derivative of yawing moment N with respect to "delta_A"

double n_delta_R;	// dimensional control derivative of yawing moment N with respect to "delta_R"

double [][] aLon = new double [4][4];					// longitudinal coefficients [A_Lon] matrix

double [][] bLon = new double [4][2];					// longitudinal control coefficients [B_Lon] matrix

double [][] aLD = new double [4][4];					// lateral-directional coefficients [A_LD] matrix

double [][] bLD = new double [4][2];					// lateral-directional control coefficients [B_LD] matrix

double[][] lonEigenvaluesMatrix = new double [4][2];		// longitudinal eigenvalues matrix

double[][] ldEigenvaluesMatrix = new double [4][2];		// lateral-directional eigenvalues matrix

RealVector eigLonVec1;	// longitudinal 1st eigenvector

RealVector eigLonVec2;	// longitudinal 2nd eigenvector

RealVector eigLonVec3;	// longitudinal 3rd eigenvector

RealVector eigLonVec4;	// longitudinal 4th eigenvector

RealVector eigLDVec1;	// lateral-directional 1st eigenvector

RealVector eigLDVec2;	// lateral-directional 2nd eigenvector

RealVector eigLDVec3;	// lateral-directional 3rd eigenvector

RealVector eigLDVec4;	// lateral-directional 4th eigenvector

double zeta_SP;         // Short Period mode damping coefficient

double zeta_PH;         // Phugoid mode damping coefficient

double omega_n_SP;      // Short Period mode natural frequency

double omega_n_PH;      // Phugoid mode natural frequency

double period_SP;       // Short Period mode period

double period_PH;      	// Phugoid mode period

double t_half_SP;	// Short Period mode halving time

double t_half_PH;	// Phugoid mode halving time

double N_half_SP;	// Short Period mode number of cycles to halving time

double N_half_PH;	// Phugoid mode number of cycles to halving time

double zeta_DR;         // Dutch-Roll mode damping coefficient

double omega_n_DR;      // Dutch-Roll mode natural frequency

double period_DR;       // Dutch-Roll mode period

double t_half_DR;	// Dutch-Roll mode halving time

double N_half_DR;	// Dutch-Roll mode number of cycles to halving time

Figure 2.11 - “Global Variables” list (part 3 of 3)

### INPUT MANAGEMENT CLASSES

The two procedures named readDataFromExcelFile and cellToString are designed for extracting and reinterpreting any information contained in specific cells of an excel file as a string value. This file has to follow a particular model or reading procedure will not succeeds. In fact, our reading procedure is designed to read only the 2nd column of the current sheet (sheet number can be selected within the main method) and to stop once reached the 47th line (as much lines as the data are).

EXAMPLE 2.10

A Correct Excel file output:

| 1 | Variable Name | Variable Value | Unit Label | Description |
| --- | --- | --- | --- | --- |
| 2 | propulsion_system | CONSTANT_TRUST | NONE | propulsion regime type |
| 3 | rho0 | 1,225 | kg*m3 | air density |
| 4 | surf | 510,97 | m^2 | wing area |
| 5 | mass | 255753 | kg | total mass |
| 6 | cbar | 8,32 | m | mean aerodynamic chord |
| 7 | bbar | 59,64 | m | wingspan |
| 8 | u0 | 85,075 | m*s | speed of the aircraft |
| 9 | m0 | 0,25 | NONE | Mach number |
| 10 | gamma0 | 0 | deg | ramp angle |
| 11 | theta0_rad | 0 | rad | Euler angle [rad] (assuming gamma0 = theta0) |
| […] | […] | […] | […] | […] |
| 46 | cNDelta_A | 0,0064 | rad^(-1) | yawing moment coefficient with respect to delta_A (CNDelta_A) of the aircraft |
| 47 | cNDelta_R | -0,109 | rad^(-1) | yawing moment coefficient with respect to delta_R (CNDelta_R) of the aircraft |
| 48 | (empty) | (empty) | (empty) | (empty) |
| 49 | (empty) | (empty) | (empty) | (empty) |

Figure 2.12 – A correct "excel file" output

The last two rows are intentionally left empty.

EXAMPLE 2.11

Figure 2.13 - Reading from excel file (part 1 of 3)

readDataFromExcelFile:

public void readDataFromExcelFile(File excelFile, int sheetNum) {

// Formats numbers up to 4 decimal places

DecimalFormat df = new DecimalFormat("#,###,##0.0000");

try {

System.out.println("Input file: " + excelFile.getAbsolutePath());

FileInputStream fis = new FileInputStream(excelFile);

Workbook wb = WorkbookFactory.create(fis);

Sheet ws = wb.getSheetAt(sheetNum);

int rowNum = ws.getLastRowNum() + 1;

System.out.println("Numero di righe: " + rowNum);

if(sheetNum == 0){

System.out.println("------------------------------------\n");

System.out.println("\n BOEING 747 /// Flight Condition (2)”);

System.out.println("____________________________________\n");

System.out.println("DATA LIST: \n");

}

else if (sheetNum == 1){

System.out.println("------------------------------------\n");

System.out.println("\n BOEING 747 /// Flight Condition (5)”);

System.out.println("____________________________________\n");

System.out.println("DATA LIST: \n");

}

for (int i = 0 ; i < rowNum ; i++) {

Row row = ws.getRow(i);

int colNum = ws.getRow(0).getLastCellNum();

for (int j = 0 ; j < colNum-2 ; j++) {

Cell cell = row.getCell(j);

String value = cellToString(cell);

switch (sheetNum){

///////////// 1st sheet /////////////

case 0:

This method extracts the rows number, using it as final value in the for cicle. During this cicle, for each row, the program reads the ith row and applies a new for cicle to read the jth column value. Then happens a switch-case cicle relative to sheet number and, right after, another switch-case cicle relative to the cell index. At this point a sub-routine saves the values in a global variable and prints it. Then the process is repeated for the 2nd sheet. If there’s any need to read a file with more than two sheets, the code can be easily modified. Moreover, code could be even more simplified by using the same routine for 1st and 2nd sheet, but we preferred to keep them separated, in case that the second sheet present a different number or order of elements.

if ((i == 1) && (j == 1)) {

propulsion_system = Propulsion.valueOf(value);

switch (propulsion_system){

case CONSTANT_TRUST:

System.out.println(" PROPULSION SYSTEM:

CONSTANT TRUST \n");

break;

[…]

default:

System.out.println(" PROPULSION SYSTEM:

CONSTANT TRUST \n");

break;

}

}

if ((i == 2) && (j == 1)) {

rho0 = Double.parseDouble(value);

System.out.println(" rho0         = " + rho0);

}

if ((i == 5) && (j == 1)) {

cbar = Double.parseDouble(value);

System.out.println(" cbar         = " + cbar);

}

[…]

if ((i == 46) && (j == 1)) {

cNDelta_R = Double.parseDouble(value);

System.out.println(" cNDelta_R    = " + cNDelta_R);

}

Figure 2.14 - Reading from excel file (part 2 of 3)

break;

Figure 2.15 - Reading from excel file (part 3 of 3)

///////////// 2nd sheet /////////////

case 0:

if ((i == 1) && (j == 1)) {

[…]

break;

}

}	}	}

catch(Exception ioe) {

ioe.printStackTrace();

}	}

The second procedure, cellToString, will not be explained in details to be lighter on the discussion but it is important to know that it provides a method for converting almost any kind of variables types into a String type.

### CONSTRUCTOR METHOD

Constructor in Java is a special type of method that is used to initialize the object.

public FlightDynamicsManager() {

}

Figure 2.16 -"Constructor" method

Java constructor is invoked at the time of object creation. It constructs the values i.e. provides data for the object that is why it is known as constructor.

### CALCULATOR METHOD

The CalculateAll method is definitely the biggest one. It has the task to recall all the other methods contained in the program and to save their results in the global variables, then it has to print them on screen to visualize the efficiency of the execution. Show in detail the entire CalculateAll structure would be unproductive, so is preferable to explain  its single steps and extract some examples from each one.

“CalculateAll” main structure (for each Longitudinal and Lateral-Directional dynamics):

- Stability and Control Derivatives Calculation;

- Prints out the Stability and Control Derivatives List;

- Generates and Prints out the A and B matrices;

- Generates and Prints out the Eigenvalues matrix of A matrix;

- Generates and Prints out the Eigenvectors of A matrix;

- Calculates and Prints out the characteristics for open-loop modes;

We will now extract some examples for each category of tasks.

EXAMPLE 2.12

Stability and Control Derivatives Calculation (X_u_CT)

Figure 2.17 -"X_u_CT" derivative calculation

x_u_CT  = StabilityDerivativesCalc.calcX_u_CT(rho0, surf, mass, u0, q0, cd0, m0, cdM0);

EXAMPLE 2.13

Stability and Control Derivatives List (X_u_CT)

// Formats numbers up to 4 decimal places

DecimalFormat df = new DecimalFormat("#,###,##0.0000");

System.out.println("LONGITUDINAL STABILITY DERIVATIVES: \n");

Figure 2.18 - "X_u_CT" derivative printing out

System.out.println(" Xªu_CT  = " + df.format(x_u_CT));

EXAMPLE 2.14

Figure 2.19 - "[A_Lon]" matrix calculation and printing out

A and B matrices (ALON matrix)

aLon = StabilityDerivativesCalc.build_A_Lon_matrix (propulsion_system, rho0, surf,

mass, cbar, u0, q0, cd0, m0, cdM0, cl0, cdAlpha0, gamma0, theta0_rad,

clAlpha0, clAlpha_dot0, cMAlpha0, cMAlpha_dot0, clQ0, iYY, cM_m0, cMq);

System.out.println(df.format(aLon[0][0])+"\t\t"+df.format(aLon[0][1])+"\t\t"+

df.format(aLon[0][2])+"\t\t"+df.format(aLon[0][3])+"\n");

[…]

System.out.println(aLon[3][0]+"\t\t"+aLon[3][1]+"\t\t"+aLon[3][2]+"\t\t"+aLon [3][3]+"\n");

EXAMPLE 2.15

Eigenvalues matrix of A matrix (ALON)

lonEigenvaluesMatrix = DynamicStabilityCalculator.buildEigenValuesMatrix(aLon);

System.out.println("  SHORT PERIOD: "+df.format(lonEigenvaluesMatrix[0][0])+" ± j"+df.format(lonEigenvaluesMatrix[0][1])+"\n");

Figure 2.20 - Longitudinal eigenvalues generation and printing out

System.out.println("  PHUGOID:      "+df.format(lonEigenvaluesMatrix[2][0])+" ± j"+df.format(lonEigenvaluesMatrix[2][1])+"\n");

EXAMPLE 2.15

Figure 2.21 - 1st longitudinal eigenvector generation and printing out

Eigenvectors of A matrix (1st Eigenvector)

System.out.println("_________________________________________________________\n");

System.out.println("LONGITUDINAL EIGENVECTORS:\n");

eigLonVec1 = DynamicStabilityCalculator.buildEigenVector(aLon, 0);

System.out.println("EigenVector 1 = " + eigLonVec1);

EXAMPLE 2.15

Figure 2.22 – “Short Period”  open-loop characteristics

Open-Loop characteristics (Short Period)

zeta_SP = DynamicStabilityCalculator.calcZeta(lonEigenvaluesMatrix[0][0],

lonEigenvaluesMatrix[0][1]);

omega_n_SP = DynamicStabilityCalculator.calcOmega_n(lonEigenvaluesMatrix[0][0],

lonEigenvaluesMatrix[0][1]);

period_SP = DynamicStabilityCalculator.calcT(lonEigenvaluesMatrix[0][0],

lonEigenvaluesMatrix[0][1]);

t_half_SP = DynamicStabilityCalculator.calct_half(lonEigenvaluesMatrix[0][0],

lonEigenvaluesMatrix[0][1]);

N_half_SP = DynamicStabilityCalculator.calcN_half(lonEigenvaluesMatrix[0][0],

lonEigenvaluesMatrix[0][1]);

System.out.println("SHORT PERIOD MODE CHARACTERISTICS\n");

System.out.println("Zeta_SP                       = "+df.format(zeta_SP)+"\n");

System.out.println("Omega_n_SP                    = "+df.format(omega_n_SP)+"\n");

System.out.println("Period                        = "+df.format(period_SP)+"\n");

System.out.println("Halving Time                  = "+df.format(t_half_SP)+"\n");

System.out.println("Number of cycles to Halving Time = "+df.format(N_half_SP)+"\n");

All customer examples cited well sums up all the statements that occur within the method. To better visualize how exactly does the Calculator works, we invite you to visualize the relative APPENDIX.

### MAIN METHOD

In the Java language, when you execute a class with the Java interpreter, the runtime system starts by calling the class's main() method. The Java Main Method then calls all the other methods required to run your application.

EXAMPLE 2.15

Java Main Method

public static void main(String[] args) {

FlightDynamicsManager theObj = new FlightDynamicsManager();

System.out.println("---------------------------------------------------");

System.out.println("Reading input data file (excel format)");

String inputFileName = "AIRCRAFT_DATA.xlsx";

File excelFile = new File (inputFileName) ;

////// select the excel sheet you want to read \\\\\\

int sheetNumber = 0;

if (excelFile.exists()){

System.out.println("File " + inputFileName + " found.");

System.out.println("\n %%% start reading from file %%% ");

// Read all data from file

theObj.readDataFromExcelFile(excelFile, sheetNumber);

System.out.println("\n %%% end of reading from file %%%");

theObj.calculateAll();

}

else {

System.out.println("File " + inputFileName + " not found.");

}

}

Figure 2.23 - Java Main Method

First step is to initialize the Constructor, then start reading from file.

Inside of it, we can select the sheet number (condition of flight). After that, Calculator starts. The Main method has been built as shortest as possible.

# BOEING 747 – TEST

We are now ready to test the program on a complete aircraft model (Boeing-747) in two specific conditions of flight, making a comparison with actual results.

## CONDITIONS OF FLIGHT

Figure 3.1 – Flight Conditions (part 1 of 2) (Heffley & Jewell, December 1972)

In the next figure, we will report the characteristics of the Boeing-747, a large airliner with turbofan engines, in two different flight conditions. The data were taken from the NASA report (Heffley & Jewell, December 1972). Numbering of the conditions considered (‘2’ and ‘5’) corresponds to the same used in the original report. All the configurations appear in a clean configuration (flap not deflected and motors in function) except for condition ‘2’, which presents the aircraft in powered approach configuration with a 20 deg flaps deflection.

| Variable | Unit | Condition 2 | Condition 5 |
| --- | --- | --- | --- |
| h0 | m | 0 | 6096 |
| ρ0 | kg/m3 | 1,225 | 0,653 |
| a0 | m/s | 340,29 | 158,02 |
| S | m2 | 510,97 | 510,97 |
| m | kg | 255753 | 288676 |
| c | m | 8,32 | 8,32 |
| b | m | 59,64 | 59,64 |
| U0 | m/s | 85,07 | 158,02 |
| q0 | kg/m∙s2 | 1098,42 | 8148,65 |
| M0 | [adim.] | 0,25 | 0,50 |
| Γ0 | deg | 0 | 0 |
| Ixx | kgm2 | 1,94107 | 2,49107 |
| Iyy | kgm2 | 4,38107 | 4,49107 |
| Izz | kgm2 | 6,14107 | 6,71107 |
| Ixz | kgm2 | -3,02106 | -3,74106 |
| SM = (Xn – Xcg)/c | [adim.] | 0,22 | 0,22 |
| B | deg | 5,70 | 6,80 |
| CD | [adim.] | 0,102 | 0,04 |

| Variable | Unit | Condition 2 | Condition 5 |
| --- | --- | --- | --- |
| CD | rad-1 | 0,66 | 0,37 |
| CDM | [adim.] | 0 | 0 |
| CL | [adim.] | 1,108 | 0,68 |
| CL | rad-1 | 5,70 | 4,67 |
| CL | rad-1 | 6,70 | 6,53 |
| CLM |  | 0 | -0,09 |
| CLq | rad-1 | 5,40 | 5,13 |
| CLδT | [adim.] | 0 | 0 |
| CLδE | rad-1 | 0,338 | 0,356 |
| Cm | rad-1 | -1,26 | -1,15 |
| Cm | rad-1 | -3,20 | -3,35 |
| CmM | [adim.] | 0 | 0,12 |
| Cmq | rad-1 | -20,80 | -20,7 |
| CmδT | rad-1 | 0 | 0 |
| CmδE | rad-1 | -1,34 | -1,43 |
| CTfix | [adim.] | 0 | 0 |
| kv | [adim.] | 0 | 0 |
| CYβ | [adim.] | -0,96 | -0,9 |
| CYp | [adim.] | 0 | 0 |
| CYr | [adim.] | 0 | 0 |
| CYδA | [adim.] | 0 | 0 |
| CYδR | [adim.] | 0,175 | 0,1448 |
| Clβ | [adim.] | -0,22 | -0,193 |
| Clp | [adim.] | -0,45 | -0,323 |
| Clr | [adim.] | 0,10 | 0,212 |
| ClδA | [adim.] | 0,046 | 0,0129 |
| ClδR | [adim.] | 0,007 | 0,0039 |
| Cnβ | [adim.] | 0,15 | 0,147 |
| Cnp | [adim.] | -0,12 | -0,0687 |
| Cnr | [adim.] | -0,30 | -0,278 |
| CnδA | [adim.] | 0,0064 | 0,0015 |
| CnδR | [adim.] | -0,109 | -0,1081 |

Figure 3.2 – Flight Conditions (part 2 of 2) (Heffley & Jewell, December 1972)

## OUTPUT

Figure 3.3 - Conditions '2' and '5' Output (part 1 of 5)

We will now see a typical output for flight conditions ‘2’ and ‘5’.

| Condition 2 | Condition 5 |
| --- | --- |
| -----------------------------------------Reading input data file (excel format) File AIRCRAFT_DATA.xlsx found. %%% start reading from file %%% Input file: C:\workspace\newproj\AIRCRAFT_DATA.xlsx rows number: 50 ----------------------------------------- BOEING 747 /// Flight Condition (2) _________________________________________ DATA LIST: PROPULSION SYSTEM: CONSTANT TRUST rho0 = 1.225 surf = 510.9667 mass = 255753 […] cNR = -0.3 cNDelta_A = 0.0064 cNDelta_R = -0.109 %%% end of reading from file %%% _________________________________________ LONGITUDINAL STABILITY DERIVATIVES: Xªu_CT = -0,0212 Xªu_CP = -0,0319 Xªw = 0,0466 Xªw_dot = 0,0000 Xªq = 0,0000 Zªu = -0,2307 Zªw = -0,6040 Zªw_dot = -0,0341 Zªq = -2,3389 Mªu = 0,0000 Mªw = -0,0064 Mªw_dot = -0,0008 Mªq = -0,4378 LONGITUDINAL CONTROL DERIVATIVES: Xªdelta_T_CT = -0,0000 Xªdelta_T_CP = -0,0000 […] Zªdelta_E = -2,9935 Mªdelta_T = 0,0000 Mªdelta_E = -0,5767 _________________________________________ MATRIX [A_LON]: -0,0212 0,0466 0,0000 -9,8100 -0,2231 -0,5841 80,0055 -0,0000 0,0002 -0,0059 -0,5011 0,0000 0.0 0.0 1.0 0.0 _________________________________________ MATRIX [B_LON]: -0,0000 0,0000 0,0000 -2,8948 0,000 -0,5744 - 0.0 _________________________________________ LONGITUDINAL EIGENVALUES SHORT PERIOD: -0,5515 ± j0,6879 PHUGOID: -0,0018 ± j0,1340 _________________________________________ LONGITUDINAL EIGENVECTORS: EigenVector 1 = {0,0062; -3,4144; -0,0940; 0,0468} EigenVector 2 = {0,8950; 10,7820; -0,0224; 0,0991} EigenVector 3 = {-41,9631; 6,24644; -0,0622; 0,7458} EigenVector 4 = {46,3490; -5,5770; 0,0991; 0,4545} _________________________________________ SHORT PERIOD MODE CHARACTERISTICS Zeta_SP = 0,6255 Omega_n_SP = 0,8816 Period = 9,1341 Halving Time = 1,2569 Number of cycles to Halving Time = 0,1376 Figure 3.4 - Conditions '2' and '5' Output (part 2 of 5) PHUGOID MODE CHARACTERISTICS Zeta_PH = 0,0132 Omega_n_PH = 0,1340 Period = 46,905 Halving Time = 391,14 Number of cycles to Halving Time = 8,3390 | ----------------------------------------- Reading input data file (excel format) File AIRCRAFT_DATA.xlsx found. %%% start reading from file %%% Input file: C:\workspace\newproj\AIRCRAFT_DATA.xlsx rows number: 50 ----------------------------------------- BOEING 747 /// Flight Condition (5) _________________________________________ DATA LIST: PROPULSION SYSTEM: CONSTANT TRUST rho0 = 0.6527 surf = 510.9667 mass = 288676 […] cNR = -0.278 cNDelta_A = 0.0015 cNDelta_R = -0.1081 %%% end of reading from file %%% _________________________________________ LONGITUDINAL STABILITY DERIVATIVES: Xªu_CT = -0,0073 Xªu_CP = -0,0110 Xªw = 0,0283 Xªw_dot = 0,0000 Xªq = 0,0000 Zªu = -0,1240 Zªw = -0,4299 Zªw_dot = -0,0157 Zªq = -1,9482 Mªu = 0,0000 Mªw = -0,0056 Mªw_dot = -0,0004 Mªq = -0,4208 LONGITUDINAL CONTROL DERIVATIVES: Xªdelta_T_CT = -0,0000 Xªdelta_T_CP = -0,0000 […] Zªdelta_E = -5,1347 Mªdelta_T = 0,0000 Mªdelta_E = -1,1040 _________________________________________ MATRIX [A_LON]: -0,0073 0,0283 0,0000 -9,8100 -0,1195 -0,4233 153,65 -0,0000 0,0003 -0,0054 -0,4870 0,0000 - 0.0 1.0 0.0 _________________________________________ MATRIX [B_LON]: -0,0000 0,0000 0,0000 -5,0554 0,0000 -1,1018 - 0.0 _________________________________________ LONGITUDINAL EIGENVALUES SHORT PERIOD: -0,4567 ± j0,9119 PHUGOID: -0,0021 ± j0,0866 _________________________________________ LONGITUDINAL EIGENVECTORS: EigenVector 1 = {0,0137; -0,0158; 0,0080; -0,0034} EigenVector 2 = {-0,0429; -1,3620; 0,0001; -0,0071} EigenVector 3 = {-89,2840; 1,0188; -0,0669; 0,5293} EigenVector 4 = {54,2225; 0,5551; 0,0442; 0,7607} _________________________________________ SHORT PERIOD MODE CHARACTERISTICS Zeta_SP = 0,4478 Omega_n_SP = 1,0199 Period = 6,8901 Halving Time = 1,5177 Number of cycles to Halving Time = 0,2203 PHUGOID MODE CHARACTERISTICS Zeta_PH = 0,0238 Omega_n_PH = 0,0866 Period = 72,555 Halving Time = 336,90 Number of cycles to Halving Time = 4,6435 |

Figure 3.5 - Conditions '2' and '5' Output (part 3 of 5)

As we can see from the free response characteristics and from the image below, it is easy to visualize the main differences between longitudinal response modes.

In particular, short period mode is characterized by a stronger damping coefficient and a higher natural frequency, which lead to a shorter period than phugoid mode.

It is also interesting to compare their values from condition ‘2’ to ‘5’, in dependence from altitude and Mach value.

Figure 3.6 - Free responses of a Boeing-747 in the longitudinal disturbances of initial condition. The two responses were obtained exciting, respectively, only the Short Period mode (left) and the Phugoid mode (right) (De Marco & Coiro, 2015)

| Condition 2 | Condition 5 |
| --- | --- |
| _________________________________________ LATERAL-DIRECTIONAL STABILITY DERIVATIVES: Yªbeta = -8,5023 Yªp = 0,0000 Yªr = 0,0000 Lªbeta = -1,5399 Lªp = -1,0992 Lªr = 0,2467 Nªbeta = 0,3299 Nªp = -0,0933 Nªr = -0,2313 LATERAL-DIRECTIONAL CONTROL DERIVATIVES: Yªdelta_A = 0,0000 Yªdelta_R = 1,5499 Lªdelta_A = 0,3212 Lªdelta_R = 0,0488 Nªdelta_A = 0,0141 Nªdelta_R = -0,2398 _________________________________________ MATRIX [A_LD]: -0,2453 0,4089 -0,0395 0,0000 -1,0000 -0,0999 0,0000 0,1153 0,2850 -1,6037 -1,0930 0,0000 - 0.0 1.0 0.0 _________________________________________ MATRIX [B_LD]: -0,0017 -0,2440 0,0000 0,0182 0,3215 0,0868 0.0 _________________________________________ LATERAL-DIRECTIONAL EIGENVALUES ROLL: -1,2306 DUTCH-ROLL: -0,0806 ± j0,7433 SPIRAL: -0,0464 Figure 3.7 - Conditions '2' and '5' Output (part 4 of 5) ________________________________________ LATERAL-DIRECTIONAL EIGENVECTORS: EigenVector 1 = {-0,1100; -0,6107; 0,7215; -0,4934} EigenVector 2 = {0,3499; -0,0873; -0,2928; -0,9171} EigenVector 3 = {0,0052; 0,1090; 1,2600; -1,0238} EigenVector 4 = {-0,3019; -0,1348; 0,1244; -2,6813} ________________________________________ DUTCH-ROLL MODE CHARACTERISTICS Zeta_DR = 0,1078 Omega_n_DR = 0,7477 Period = 8,4529 Halving Time = 8,5975 Number of cycles to Halving Time = 1,0171 | _________________________________________ LATERAL-DIRECTIONAL STABILITY DERIVATIVES: Yªbeta = -12,9810 Yªp = 0,0000 Yªr = 0,0000 Lªbeta = -1,9212 Lªp = -0,6068 Lªr = 0,3983 Nªbeta = 0,5439 Nªp = -0,0480 Nªr = -0,1941 LATERAL-DIRECTIONAL CONTROL DERIVATIVES: Yªdelta_A = 0,0000 Yªdelta_R = 2,0885 Lªdelta_A = 0,1284 Lªdelta_R = 0,0388 Nªdelta_A = 0,0056 Nªdelta_R = -0,4000 _________________________________________ MATRIX [A_LD]: -0,2182 0,6566 -0,0143 0,0000 -1,0000 -0,0822 0,0000 0,0621 0,4310 -2,0197 -0,6047 0,0000 - 0.0 1.0 0.0 _________________________________________ MATRIX [B_LD]: -0,0016 -0,4056 0,0000 0,0132 0,1287 0,0997 - 0.0 _________________________________________ LATERAL-DIRECTIONAL EIGENVALUES ROLL: -0,7414 DUTCH-ROLL: -0,0729 ± j0,8562 SPIRAL: -0,0179 _________________________________________ LATERAL-DIRECTIONAL EIGENVECTORS: EigenVector 1 = {0,2651; 0,2718; -0,8138; 0,0830} EigenVector 2 = {-0,1770; 0,3066; 0,0023; 0,9435} EigenVector 3 = {0,0623; -0,0794; -1,3701; 1,8480} EigenVector 4 = {-0,2514; -0,0751; 0,0738; -4,1280} _________________________________________ DUTCH-ROLL MODE CHARACTERISTICS Zeta_DR = 0,0848 Omega_n_DR = 0,8593 Period = 7,3387 Halving Time = 9,5143 Number of cycles to Halving Time = 1,2964 |

Figure 3.8 - Conditions '2' and '5' Output (part 5 of 5)

Figure 3.9 - Free responses of a Boeing-747 in the lateral-directional disturbances of initial condition. The two responses were obtained exciting, respectively, only the Roll mode (left) and the Spiral mode (right) (De Marco & Coiro, 2015)

The free responses shown in Figures 3.6, 3.9 and 3.10 are obtained resolving the problem for an initial condition coincident, respectively, with the (left) eigenvector of free response modes.

Figure 3.10 - Free response of a Boeing 747 in lateral-directional perturbation, obtained exciting just the Dutch-Roll mode (De Marco & Coiro, 2015)FINAL REMARKS

## FINAL REMARKS

Stability analysis transcends our discussion, in fact our program simply extrapolates dynamics key values without analyzing them in a system response optics. Nonetheless, it is immediate to note how all the eigenvectors are characterized by a negative real part, which supports the stability condition for small perturbations hypothesis. It is also clear how the values between condition '2' and '5' are comparable in order of magnitude. In addition, the differences between damping coefficient and natural frequency in longitudinal response modes explains why we easily distinguish the short period and phugoid modes according to their dynamic characteristics. Moreover, plotting damping coefficient in terms of natural frequency is of theoretical interest to determine a zone of response optimization, according to the Thumb Print criterion (De Marco & Coiro, 2015).

Finally, we recall that all the results achieved are in exact form, obtained directly from the eigenvalues. There are also approximation formulas obtained instead manipulating the stability derivatives, which are more or less valid depending on the conditions of motion. Some cases, in particular the dutch-roll mode, are difficult to approximate, since they induce variations on all four state variables. That is why, for the purpose of a more appropriate and applicable analysis, we do not use these formulas to derive the characteristics of response modes but we prefer, instead, to obtain them from a more complex and accurate study on eigenvalues.

# BIBLIOGRAPHY

Apache Commons. (s.d.). Commons Math: The Apache Commons Mathematics Library, 3.5. Tratto da https://commons.apache.org/proper/commons-math/

De Marco, A., & Coiro, D. P. (2015, Maggio). Elementi di Dinamica e Simulazione di Volo - Quaderno 16 (Piccole Perturbazioni del Moto di un Velivolo). Tratto da http://wpage.unina.it/agodemar/DSV-DQV/#materiale-didattico: http://wpage.unina.it/agodemar/DSV-DQV/DSV-DQV_Quaderno_16.pdf

Heffley, R. K., & Jewell, W. F. (December 1972). Aircraft Handling Qualities Data. NASA-CR-2144.

Sharan, K. (2014). Beginning Java 8 Fundamentals: Language Syntax, Arrays, Data Types, Objects, and Regular Expressions. Apress.

# NOTES

Real roots λ HEIGHT and λ RANGE do not appear in our characteristic polynomial because matrix ALON order has been reduced due to our simplified nominal condition. Their weight would have been negligible, anyway.